Thank you for downloading the Dynamite® Passport™ Ultra Firmware Upgrade tool. With this software you can update the firmware in your Dynamite Passport Ultra charger with exciting new features and benefits. For ease of access we recommend you place the folder with the files you downloaded onto your desktop. In this download you’ll have the following items:

Firmware Upgrade Tool – This is the user interface you will use to update your charger’s firmware.

Windows Drivers – You will find the appropriate 32-bit and 64-bit Windows drivers.

DYNCXXXXXXXXX.fmw – This is the firmware file you will be applying to your charger.

Step 1: Open the Firmware Upgrade Tool folder and double click on the Balance Charger Upgrade Setup.exe file. Follow the on-screen installation prompts.

Step 2: Once the installation is complete, check the box labeled “Launch CP210 Drivers Setup” and click “Finish”. This will launch the USB to UART Bridge Driver Installer software. Follow the on-screen prompts

Step 3: If you’ve successfully installed both the software and the drivers, you will see a “Balance Charger Upgrade vX.XX (where the Xs represent the version of the software) in your Start Menu. Click on this to launch the software.

Step 4: Within the software, click the “Open” button. This will open a window where you will navigate to the folder you downloaded the .fmw firmware file. Double click on this file.

Step 5: With your charger powered off, connect your charger to your computer via USB cable. DO NOT APPLY POWER YET! Once the computer detects your charger, the Port section will populate a value to indicate the port the charger is connected to. Once that value is displayed, click the “Upgrade” button to begin the process.

Step 6: When prompted, the software will indicate that it is starting the communication process with your charger and attempt a handshake between your computer and charger. At this point connect your charger to a power source. When connected, you will see an on-screen message that the update has started and to please wait.

Step 7: When the update is complete you will see a message stating “Update & verify OK”. Your charger should power cycle at this point. If it does not, manually disconnect the charger from both the computer and power source, wait 10-seconds, and then reconnect the power cable.

Step 8: Verify the new features have been added via the firmware update. You can verify that new features have been added by navigating the menu system.